Setup sheet for the LS4000 SERIES RS232 scanners

Before you begin scanning the bar codes, plug the scanner base into a power outlet and let the scanner charge completely.

Once the scanner is charged, pair the scanner with the base by scanning the bar code on the base.

Scan:	Set All Defaults

Scan:	Set Synapse RS232 Defaults

Scan:	Baud Rate 19,200

Scan:	Female Host Connector/Male Local Connector

Scan:	TXD Pin 2 Host/TXD Pin 3 Local

Scan:	Baud Rate 19,200

Scan:	Bar codes numbers 7-16

Scan:	Scan Options

Scan:	<Data><Suffix>

Scan:	Enter

Scan:	Scan Suffix

Scan:	1

Scan:	0

Scan:	1

Scan:	3

Scan:    Single Line Only

Make sure the keyboard works then try to scan something in stock status. If it does nothing, unplug the scanner base and plug back in.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

15

16

17

18

19

20

21

22

23

24